[image: image48][image: image49]
[image: image48]

 SHAPE  \* MERGEFORMAT 
[image: image49]

Built-In Hob
OM 752 EDWI
Please read this manual first.
Dear Customer, 
We would like you to obtain best performance with your product that has been manufactured in modern facilities and passed through strict quality control procedures.
Therefore, we advise you to read through this User Manual carefully before using your product and keep it for future reference. If you hand over the product to someone else, deliver the User Manual as well.
The User Manual will help you use the product in a fast and safe way.
· Read the User Manual before installing and operating the product.
· You must strictly observe the safety instructions.
· Keep this User Manual within easy reach for future reference.
· Also read other documents supplied with the product.
Remember that this User Manual may also be applicable for several other models.
Differences between models are explicitly identified in the manual.
Explanation of symbols
Throughout this User Manual the following symbols are used:
[image: image1.png]


Important information or useful hints about usage.
[image: image2.png]


Warning for hazardous situations with regard to life and property.
[image: image3.png]


Warning for electric shock.
[image: image4.png]


Warning for risk of fire.
[image: image5.png]


Warning for hot surfaces.
Table of Contents
41 General information


4Overview


4Technical specifications


5Package contents


62 Important safety instructions


6General safety


6Safety for children


6Safety when working with gas


7Intended use


83 Installation


8Before installation


9Installation and connection


11Compliance with EEE Directive


11Package information


12Future Transportation


12Disposing of the old product


134 Preparation


13Tips for saving energy


13Initial use


13First cleaning of the appliance


145 How to use the hob


14General information about cooking


14Using the hobs


166 Maintenance and care


16General information


16Cleaning the hob


177 Troubleshooting


188 Consumer Service


18ingilizce versiyonu yoktur


199 Ankastre ocak montaj bilgilendirme formu


19silinecek




1 General information

Overview
[image: image6.png]



	1
	Pan support

	2
	Auxiliary burner

	3
	Assembly clamp

	4
	Control panel

	5
	Base cover

	6
	Wok burner

	7
	Burner plate

	8
	Normal burner

	9
	Normal burner


[image: image7.png]



	1
	Normal burner Rear left

	2
	Wok burner Front left

	3
	Auxiliary burner Front right

	4
	Normal burner Rear right


Technical specifications
	Voltage / frequency
	220-230 V ~ 50 Hz

	Cable type / section
	H05RR-FG
3 x 0,75 mm2

	Cable length
	max. 2 m

	Category of gas product
	II 2H3B/P 

	Gas type / pressure
	NG G 20/20 mbar

	Total gas consumption
	7.8 kW

	Converting the gas type / pressure§
	LPG G 30/28-30 mbar

	– optional
	


	External dimensions (height / width / depth)
	99 mm/580 mm/510 mm

	Installation dimensions (width / depth)
	560 mm/480-490 mm


	Burners

	Rear left
	Normal burner

	Power
	1750 W    

	Front left
	Wok burner

	Power
	3300 W    

	Front right
	Auxiliary burner

	Power
	1000 W    

	Rear right
	Normal burner

	Power
	1750 W    


	[image: image8.png]



	Technical specifications may be changed without prior notice to improve the quality of the product.


	[image: image9.png]



	Figures in this manual are schematic and may not exactly match your product.


	[image: image10.png]



	Values stated on the product labels or in the documentation accompanying it are obtained in laboratory conditions in accordance with relevant standards. Depending on operational and environmental conditions of the product, these values may vary.


Package contents
	[image: image11.png]



	Accessories supplied can vary depending on the product model. Not every accessory described in the user manual may exist on your product.


User manual
Coffee pot adaptor
[image: image12.png]



Used for coffe pots.
Vok pan adaptor
[image: image13.png]



Used for hosting a wok pan with a round bottom.
2 Important safety instructions

This section contains safety instructions that will help you avoid risk of injury and damage. All warranties will become void if you do not follow these instructions. 
General safety
· This product is not intended to be used by persons with physical, sensory or mental disorders or unlearned or inexperienced people (including children) unless they are attended by a person who will be responsible for their safety or who will instruct them accordingly for use of the product.
Children must be supervised to prevent them from tampering with the product.
· Connect the product to an earthed outlet/line protected by a fuse of suitable capacity as stated in the "Technical specifications" table. Have the earthing installation made by a qualified electrician while using the product with or without a transformer. Our company shall not be liable for any problems arising due to the product not being earthed in accordance with the local regulations.
· If the supply cord is damaged, it must be replaced by the manufacturer, its service agent or similary qualified persons in order to avoid a hazard.
· Do not operate the product if it is defective or has any visible damage.
· Do not carry out any repairs or modifications on the product. However, you may remedy some troubles that do not require any repairs or modifications. See Troubleshooting, page 17.
· Never wash down the product with water! There is the risk of electric shock!
· Never use the product when your judgment or coordination is impaired by the use of alcohol and/or drugs.
· Product must be disconnected from the mains during installation, maintenance, cleaning and repair procedures.
· Be careful when using alcoholic drinks in your dishes. Alcohol evaporates at high temperatures and may cause fire since it can ignite when it comes into contact with hot surfaces.
· Do not heat closed tins and glass jars in the oven. The pressure that would build-up in the tin/jar may cause it to burst.
· Do not place any flammable materials close to the product as the sides may become hot during use.
· Keep all ventilation slots clear of obstructions.
· Make sure that the product function knobs are switched off after every use.
Safety for children
· Electrical and/or gas products are dangerous to children. Keep children away from the product when it is operating and do not allow them to play with the product.
· Accessible parts of the appliance will become hot whilst the appliance is in use. Children should be kept away until the product has cooled down.
· Do not place any items above the appliance that children may reach for.
· The packaging materials will be dangerous for children. Keep the packaging materials away from children. Please dispose of all parts of the packaging according to environmental standards.
SAVE THESE INSTRUCTIONS
Safety when working with gas
· Any works on gas equipment and systems may only be carried out by authorised qualified persons who are Gas Safe registered.
· "Prior to the installation, ensure that the local distribution conditions (nature of the gas and gas pressure) and the adjustment of the appliance are compatible."
· "This appliance is not connected to a combustion products or evacuation device. It shall be installed and connected in accordance with current installation regulations. Particular attention shall be given to the relevant requirements regarding ventilation;" See Before installation, page 8 
· "CAUTION: The use of gas cooking appliance results in the production of heat and moisture in the room in which it is installed. Ensure that the kitchen is well ventilated: keep natural ventilation holes open or install a mechanical ventilation device (mechanical extractor hood). Prolonged intensive use of the appliance may call for additional ventilation, for example increasing the level of the mechanical ventilation where present."
· Gas appliances and systems must be regularly checked for proper functioning. Regulator, hose and its clamp must be checked regularly and replaced within the periods recommended by its manufacturer or when necessary.
· Clean the gas burners regularly. The flames should be blue and burn evenly.
· Good combustion is required in gas appliances. In case of incomplete combustion, carbon monoxide (CO) might develop. Carbon monoxide is a colourless, odourless and very toxic gas, which has a lethal effect even in very small doses.
· Request information about gas emergency telephone numbers and safety measures in case of gas smell from you local gas provider.
What to do when you smell gas
	[image: image14.png]



	DANGER
Risk of explosion and poisoning!
Put out all naked flames and do not smoke, strike matches or do anything that could cause ignition.
Do not operate any electric switches (e.g. light switch, TV, computer, cooker or door bell).
Do not use a telephone or mobile phone.


1.
Turn off all valves on gas appliances and gas meter at the main control valve, unless it's in a confined space or cellar.
2.
Open doors and windows.
3.
Check all tubes and connections for tightness. If you still smell gas leave the property.
4.
Warn the neighbours.
5.
Call the fire-brigade. Use a telephone outside the house.
5.1
If you have door or gates that are secured by an electric lock, or entry phone, please open these manually.
6.
Do not re-enter the property until you are told it is safe to do so.
Intended use
This product is designed for domestic use. Commercial use will void the guarantee.
"CAUTION: This appliance is for cooking purposes only. It must not be used for other purposes, for example room heating."
The manufacturer shall not be liable for any damage caused by improper use or handling errors.
Service life of your product is 10 years. Manufacturer shall provide the required spare parts for the product to perform as defined during this period. 
3 Installation

Refer to the nearest Authorised Service Agent for installation of the product. Make sure that the electric and gas installations are in place before calling the Authorised Service Agent to have the product ready for operation. If not, call a qualified electrician and fitter to have the required arrangements made. The manufacturer shall not be held responsible for damages arising from procedures carried out by unauthorized persons which may also void the warranty.
	[image: image15.png]



	Preparation of location and electrical and gas installation for the product is under customer’s responsibility.


	[image: image16.png]



	DANGER
The product must be installed in accordance with all local gas and/or electrical regulations.


	[image: image17.png]



	DANGER
Prior to installation, visually check if the product has any defects on it. If so, do not have it installed.
Damaged products cause risks for your safety.


Before installation
The hob is designed for installation in a commercial worktop. A safety distance must be left between the appliance and the kitchen walls and furniture. See figure (values in mm).
· It can also be used in a free standing position. Allow a minimum distance of 750 mm above the hob surface.
· (*) If a cooker hood is to be installed above the cooker, refer to cooker hood manufacturer' instructions regarding installation height (min 650 mm)
· Remove packaging materials and transport locks.
· Surfaces, synthetic laminates and adhesives used must be heat resistant (100 °C minimum).
· The worktop must be aligned and fixed horizontally.
· Cut aperture for the hob in worktop as per installation dimensions.
[image: image18.png]510





Room ventilation
Good room ventilation is essential for safe operation of your appliance. If there is no window or door available for room ventilation, an extra ventilation must be installed.
	Room size
	Ventilation opening

	smaller than 5  m3
	min. 100 cm2

	5 m3 to 10 m3
	min. 50 cm2

	greater than 10  m3'den 
	not required

	In the cellar or basement
	min. 65 cm2


Installation and connection
· The product must be installed in accordance with all local gas and electrical regulations.
Electrical connection
Connect the product to a grounded outlet/line protected by a fuse of suitable capacity as stated in the "Technical specifications" table. Have the grounding installation made by a qualified electrician while using the product with or without a transformer. Our company shall not be liable for any damages that will arise due to using the product without a grounding installation in accordance with the local regulations.
	[image: image19.png]



	DANGER
Risk of electric shock, short circuit or fire by unprofessional installation!
The appliance may be connected to the mains supply only by an authorized qualified person and the appliance is only guaranteed after correct installation.


	[image: image20.png]



	DANGER
Risk of electric shock, short circuit or fire by damage of the mains lead!
The mains lead must not be clamped, bent or trapped or come into contact with hot parts of the appliance.
If the mains lead is damaged, it must be replaced by a qualified electrician.


· Connection must comply with national regulations.
· The mains supply data must correspond to the data specified on the type label of the product. Type label is at the rear housing of the product.
· Power cable of your product must comply with the values in "Technical specifications" table.
	[image: image21.png]



	DANGER
Risk of electric shock!
Before starting any work on the electrical installation, please disconnect the appliance from the mains supply.


	[image: image22.png]



	Power cable plug must be within easy reach after installation (do not route it above the hob).


Plug the power cable into the socket.
Gas connection
	[image: image23.png]



	DANGER
Risk of explosion or poisoning by unprofessional installation.
The appliance may be connected to the gas distribution system only by an authorized qualified person who is Gas Safe registered.


	[image: image24.png]



	DANGER
Risk of explosion!
Before starting any work on the gas installation, please shut off the gas supply.


· Gas adjustment conditions and values are stated on labels (or ion type label).
	[image: image25.png]



	Your appliance is adjusted for Natural gas.


· Natural gas installation must be prepared before installing the product.
· There must be a flex pipe at the outlet of the installation for the natural gas connection of your product and the tip of this pipe must be closed with a blind plug. Service Representative will remove the blind plug during connection and connect your product by means of a straight connection device (coupling).
· Make sure that the natural gas valve is readily accessible.
· If you need to use your product later with a different type of gas, you must consult the Authorised Service Agent for the related conversion procedure.
If there is a built-in oven installed under the hob;
· Gas hose must be routed in a way not to touch the sharp edges and corners beneath the hob and not to be kinked or twisted.
If you will make the gas connection from left hand side when viewed from your position, fix the hose with clamps.
[image: image26.png]



	1
	Hob (appearance from below)

	2
	Hose tip

	3
	Hose

	4
	Clamp

	5
	Oven


Installing the product

1.
Remove burners, burner caps and grills on the hob from the product.

2.
Turning the hob upside down, place it on a flat  

      surface.

3.
In order to avoid the foreign substances and liquids penetrate between the hob and the counter, please apply the putty supplied in the package on the contours of the lower housing of the hob but make sure that the putty does not overlap.

4.
Applied the putty with bending at the corners like in the figure. Increase the bends to the filling the spaces. 

[image: image27.png]



	[image: image28.png]



	Make sure  the putty is filled on base cover contours.


5.
Turning the hob again, align and place it on the counter.

6.
You can secure your hob to the counter by using the clamp and the screws supplied.

[image: image29.png]



	*
	min. 20 & max. 40 mm

	1
	Hob

	2
	Screw

	3
	Installation clamp

	4
	Counter

	5
	Putty


Rear view (connection holes)
[image: image30.png]



Place the burner plates, burner plate caps and grills back to their seating after installation.
	[image: image31.png]



	Making connections to different holes is not a good practice in terms of safety since it can damage the gas and electrical system.


Final check
1.
Open gas supply.
2.
Check gas installations for secure fitting and tightness.
3.
Ignite burners and check appearance of the flame.
	[image: image32.png]



	The flames should be blue and burn smoothly. If the flames appear yellowish, check secure position of the burner cap or clean the burner. Switch off gas and let burner cool before touching it.


Compliance with EEE Directive
This product does not contain harmful and forbidden materials described in the "Directive on the Restriction of the Use of Certain Hazardous Substances in Electrical and Electronic Equipment" (EEE) issued by the T.R. Ministry of Environment and Forest.
Complies with the EEE Directive.
Package information
Packaging materials are dangerous to children. Keep packaging materials in a safe place out of reach of children.
Packaging materials of the product are manufactured from recyclable materials in accordance with our National Environment Regulations.
Do not dispose of the packaging materials together with the domestic or other wastes. Take them to the packaging material collection points designated by the local authorities.
Future Transportation
· Keep the product's original carton and transport the product in it. Follow the instructions on the carton. If you do not have the original carton, pack the product in bubble wrap or thick cardboard and tape it securely.
· Secure the caps and pan supports with adhesive tape.
	[image: image33.png]



	Check the general appearance of your product for any damages that might have occurred during transportation.


Disposing of the old product
[image: image34.png]



Dispose of the old product in an environmentally friendly manner.
This equipment bears the selective sorting symbol for waste electrical and electronic equipment (WEEE). This means that this equipment must be handled pursuant to European Directive 2002/96/EC in order to be recycled or dismantled to minimize its impact on the environment. For further information, please contact local or regional authorities. 
Refer to your local dealer or solid waste collection centre in your area to learn how to dispose of your product.
Before disposing of the product, cut off the power cable plug and make the door lock (if any) unusable to avoid dangerous conditions to children.
4 Preparation

Tips for saving energy
The following information will help you to use your appliance in an ecological way, and to save energy:
· Defrost frozen dishes before cooking them.
· Use pots/pans with cover for cooking. If there is no cover, energy consumption may increase 4 times.
· Select the burner which is suitable for the bottom size of the pot to be used. Always select the correct pot size for your dishes. Larger pots require more energy.
Initial use
First cleaning of the appliance
	[image: image35.png]



	The surface may get damaged by some detergents or cleaning materials.
Do not use aggressive detergents, cleaning powder/cream or any sharp objects.


1.
Remove all packaging materials.
2.
Wipe the surfaces of the appliance with a damp cloth or sponge and dry with a cloth.
5 How to use the hob

General information about cooking
	[image: image36.png]



	DANGER
Risk of fire by overheated oil!
When you heat oil, do not leave it unattended.
Never try to extinguish the fire with water! When oil has caught fire, immediately cover the pot or pan with a lid. Switch off the hob if safe to do so and phone the fire service.


· Before frying foods, always dry them well and gently place into the hot oil. Ensure complete thawing of frozen foods before frying.
· Do not cover the vessel you use when heating oil.
· Place the pans and saucepans in a manner so that their handles are not over the hob to prevent heating of the handles. Do not place unbalanced and easily tilting vessels on the hob.
· Do not place empty vessels and saucepans on cooking zones that are switched on. They might get damaged.
· Operating a cooking zone without a vessel or saucepan on it will cause damage to the product. Turn off the cooking zones after the cooking is complete.
· As the surface of the product can be hot, do not put plastic and aluminum vessels on it.
Such vessels should not be used to keep foods either.
· Use flat bottomed saucepans or vessels only.
· Put appropriate amount of food in saucepans and pans. Thus, you will not have to make any unnecessary cleaning by preventing the dishes from overflowing.
Do not put covers of saucepans or pans on cooking zones.
Place the saucepans in a manner so that they are centered on the cooking zone. When you want to move the saucepan onto another cooking zone, lift and place it onto the cooking zone you want instead of sliding it.
Gas cooking 
· Size of the vessel and the flame must match each other. Adjust the gas flames so that they will not extend the bottom of the vessel and center the vessel on saucepan carrier.
Using the hobs
[image: image37.png]



	1
	Normal burner 16-18 cm

	2
	Wok burner 20 -22 cm 

	3
	Auxiliary burner 12-16 cm 

	4
	Normal burner 16-18 cm is list of advised diameter of pots to be used on related burners.


Large flame symbol indicates the highest cooking power and small flame symbol indicates the lowest cooking symbol. In turned off position (top), gas is not supplied to the burners.
Igniting the gas burners
1.
Keep burner knob pressed.
2.
Turn it counter clockwise to large flame symbol.
» Gas is ignited with the spark created.
3.
Adjust it to the desired cooking power.
Turning off the gas burners
Turn the keep warm zone knob to off (upper) position.
Gas shut off safety system (in models with thermic component)
	[image: image38.png]



1. Gas shut off safety
	As a counter measure against blow out due to fluid overflows at burners, safety mechanism trips and shuts off the gas.


· Push the knob inwards and turn it counter clockwise to ignite.
· After the gas ignites, keep the knob pressed for 3-5 seconds more to engage the safety system. 
· If the gas does not ignite after you press and release the knob, repeat the same procedure by keeping the knob pressed for 15 seconds. 
	[image: image39.png]



	DANGER
Risk of deflagration incase of unburned gas!
Do not press the knob for longer than 15 seconds if there was no ignition.
Wait for one minute and then repeat the ignition process.


Wok burner
[image: image40.png]



Wok burners help you to cook faster. Wok, which is particularly used in Asian kitchen is a kind of deep and flat fry-pan made of sheet metal, which is used to cook minced vegetable and meat at strong flame in a short time.
Since meals are cooked at strong flame and in a very short time in such fry-pans that conduct the heat rapidly and evenly, the nutrition value of the food is preserved and vegetables remain crispy.
You can use wok burner for regular saucepans as well.
If you want to use regular saucepan on wok burner, you must take out the wok fry-pan carrier from the hob.
6 Maintenance and care

General information
Service life of the product will extend and the possibility of problems will decrease if the product is cleaned at regular intervals.
	[image: image41.png]



	DANGER
Risk of electric shock!
Switch off the electricity before cleaning appliance to avoid an electric shock.


	[image: image42.png]



	DANGER
Hot surfaces may cause burns!
Allow the appliance to cool down before you clean it.


· Clean the product thoroughly after each use. In this way it will be possible to remove cooking residues more easily, thus avoiding these from burning the next time the appliance is used.
· No special cleaning agents are required for cleaning the product. Use warm water with washing liquid, a soft cloth or sponge to clean the product and wipe it with a dry cloth.
· Always ensure any excess liquid is thoroughly wiped off after cleaning and any spillage is immediately wiped dry.
· Do not use cleaning agents that contain acid or chloride to clean the stainless or inox surfaces and the handle. Use a soft cloth with a liquid detergent (not abrasive) to wipe those parts clean, paying attention to sweep in one direction.
	[image: image43.png]



	The surface may get damaged by some detergents or cleaning materials.
Do not use aggressive detergents, cleaning powder/cream or any sharp objects.


	[image: image44.png]



	Do not use steam cleaners for cleaning.


Cleaning the hob
Gas hobs
1.
Remove and clean the saucepan carriers and burner caps.
2.
Clean the hob.
3.
Install the burner caps and make sure that they are seated correctly.
4.
When installing the upper grills, pay attention to place the saucepan carriers so that the burners are centered.
[image: image45.png]N





[image: image46.png]



7 Troubleshooting

	There is no gas.

	Main gas valve is closed. 
	Open gas valve.

	Gas pipe is bent.
	Install gas pipe properly.

	There is no ignition spark. 

	No current. 
	Check fuses in the fuse box.


	Burners are not burning properly or at all.

	Burners are dirty.
	Clean burner components.

	Burners are wet.
	Dry the burner components.

	Burner cap is not mounted safely.
	Mount the burner cap properly.

	Gas valve is closed.
	Open gas valve.

	Gas cylinder is empty (when using LPG). 
	Replace gas cylinder.


	[image: image47.png]



	Consult the Authorised Service Agent or the dealer where you have purchased the product if you can not remedy the trouble although you have implemented the instructions in this section. Never attempt to repair a defective product yourself.


	


	185924633
	17.04.2012
	R.AA



[image: image50.png]


[image: image51.png]


[image: image52.emf]